SEXTANT NAVIGATOR · FULL PROCEDURE

# From Sextant to Fix

The whole run, from the parking spot to the fix plotted on the chart. The work is split between two stations, as it was: the instrument under the astrodome, the table at the navigator’s position. Every step says which of the two does it.

SEXTANT  the panel, inside the simulator        TABLE  Sextant Navigator, alongside

The panel speaks French. Sextant Navigator switches between French and English; the sextant panel does not — its engraved legends are French and stay French. This document gives them as they appear on screen: VISER to start the integration, ARRETER to stop it, TABLE / TABLE HORS LIGNE for the bridge. Everything else below is the application’s own English wording.

# BEFORE THE FLIGHT — ON THE GROUND

Calibration happens here and nowhere else: it is the only moment when you truly know where you are.

## 01   TABLE   Start the table and hook it to the simulator

Open Sextant Navigator, then Connect MSFS2024.

The table reads position, heading, airspeed and zulu time through SimConnect. The panel asks it for none of that: the two halves never tread on each other.

## 02   SEXTANT   Open the sextant inside the simulator

MSFS toolbar → the sextant panel. The top band carries the state of the bridge: TABLE in green once it is taken, TABLE HORS LIGNE otherwise.

The panel opens a WebSocket on ws://127.0.0.1:8787. That is the only possible direction — an MSFS panel can open a connection, it cannot receive one. If the table is not there, the sextant works alone and keeps its sights.

## 03   TABLE   Declare your true airspeed and the forecast wind

Show flight plan panel → the TAS and Forecast wind boxes.

That is the wind your dead reckoning will run on. The simulator applies another one, and you do not know it: the gap between the two is exactly what the line of position will reveal. With no wind declared, the DR runs in still air and drifts all the more.

## 04   TABLE   Open a calibration run

Sights received button on the bar → Calibration, at the far right of the line → From the airfield.

On the ground the DR is worth the true position — you know which airfield you took off from, and that is the one legitimate channel. You may also type a latitude and longitude by hand. Airborne it is refused: you would measure the drift of your DR instead of the instrument.

## 05   TABLE   Pick SLOW bodies

Star catalogue button → the ′/min column. Take the ones under 3′/min, in green. Those are the bodies near the meridian — due north or due south.

Click the send button on the row: the body goes to the sextant as an order.

The rule that decides everything. A calibration run does not only measure the sextant: it also measures your knob lag. On a body descending at 10′/min with a drum half a minute behind, you read five arcminutes too high — and you charge them to the instrument. That is the mistake actually made in flight on 21 August.

## 06   SEXTANT   Shoot six to ten times

The shooting procedure is that of steps 11 to 17, unchanged. Six for the table to conclude, ten for a full run.

Each sight leaves for the table on its own as soon as you stop the integration.

## 07   TABLE   Measure, then adopt

In the calibration block: Measure. The table gives, sight by sight, Ho − Hc and the body’s rate. Then Adopt.

The correction drops into the “Index error” box of the book, is written to disk, and comes back at the next start. A sextant calibrated yesterday still is.

The table also states how it concluded: on slow bodies (the sound method), on the run’s slope (the recovery), or on the raw mean (the last resort, which mixes the instrument with the hand). If the run has leverage, it measures your knob lag in seconds along the way — that is your hand, not the instrument, and it can be fixed on the very next sight.

# AIRBORNE — PREPARING THE ROUND

## 08   TABLE   See what the sky offers

Star catalogue button. Six columns matter:

Hc the altitude, to set on the drum · RB the relative bearing, to set on the crown · Zn the true azimuth, for the plot and the cut · ′/min the vertical rate · Mag the brightness · Cut the crossing angle.

Hc and RB go on the instrument, Zn does not. The sextant’s crown is fixed to the airframe: it is graduated in relative bearing and knows nothing of north. The two columns that go on the instrument form a tinted block closed by a blue rule — it is there so you cannot mistake them.

## 09   TABLE   Choose three bodies

The band proposes the best trio and the best pair, with their cut and their dilution. You may also tick your own: the cut column updates and gives the worst pair of your selection.

Dilution is the factor by which one sight’s error is multiplied on the chart. Two bodies 20° apart multiply it by four. A line also says what was left out of the 15°–75° domain and by how much — a body half a degree below the limit is of interest to you.

Shoot them close together in time. At 150 kt, ten minutes between the first and the last sight make twenty-five miles of run. The transport catches them up, but it does so along the track you *believe* you flew: the more the round spreads out, the more your wind error contaminates the fix.

## 10   TABLE   Pass the order

The send button on the body’s row. Clicking the same body again cancels the order and empties the sextant’s field.

The catalogue precomputes from your dead reckoning, never from your true position. So the body is not quite where you are told it is: thirty miles of DR error move it half a degree in relative bearing. Enough to have to look for it, never enough to lose it.

# AT THE SEXTANT — THE SIGHT

To be repeated for each body: a fresh order is needed every time.

## 11   SEXTANT   Read the order plate

At the top of the panel: VEGA · Gt 170° · h 38° — Gt is the relative bearing, h the altitude.

It does not refresh. It is a slip of paper handed to you, and it ages while the aircraft turns. If you have banked since, the bearing is no longer good — it is for the navigator to pass another one.

## 12   SEXTANT   Set the bearing and the altitude

Bearing crown: coarse knob, then fine to a tenth of a degree. Altitude drum: coarse knob, then fine to the arcminute. The window counter shows both.

The drum runs from −10° to +92°. The knobs’ step is not a setting: it falls out of their on-screen diameter and their units per turn, as on the real instrument.

## 13   SEXTANT   Search at ×1

Magnification selector on ×1: fifteen degrees of field. Eight-position filter wheel if there is still daylight. Bubble knob for its size.

By day a filter is mandatory to see the bubble at all — it is in the manual, and it is reproduced. The scale at the bottom of the field gives the true azimuth of what you are aiming at: collimate the Sun and you read your true heading. That is the astro-compass, free.

## 14   SEXTANT   Collimate at ×5

Switch to ×5: three degrees of field. Bring the body to the centre of the bubble.

At ×1 one pixel is worth 2.1 arcminutes — the 2′ the instrument claims is physically out of reach there. At ×5 it is worth 0.4′. That is why there are two magnifications and not one.

## 15   SEXTANT   Press VISER

The plunger goes down, the amber lamp comes on, the legend changes to ARRETER.

## 16   SEXTANT   Integrate, following the body on the knob

Thirty seconds at least, two minutes at most — past that, the integration stops on its own.

This is where the accuracy is won or lost. The integrator averages the position of the drum, not the truth of the sky. If the body descends and the knob does not follow, your mean is too high by just that much. The ′/min column told you in advance how much cranking it would take: under 3 you can set it and let it run; over 8 you must follow continuously, or shoot shorter.

## 17   SEXTANT   Press ARRETER

The sight leaves for the table: the mean altitude, the mid-time of the integration, its length, and the flight conditions of the moment.

A sight under thirty seconds is rejected and the panel says so. If the table is not connected, the sight stays in the queue and will leave at reconnection — it is forgotten only against an acknowledgement, and the acknowledgement comes only once the sight is written to disk.

## 18   SEXTANT   Do it again for the other two bodies

Back to step 10: fresh order, fresh sight.

# AT THE TABLE — THE FIX

## 19   TABLE   Check the book

Sights received button. Each line: date, Mid-time, body, raw Hs, Length, Alt.

The Hs stays raw — no index error, no refraction, no parallax. Showing it as an observed altitude would lie by a good ten arcminutes. A length under one minute shows as a warning: the sight is jittery.

## 20   TABLE   Check the index error

The Index error box carries the correction adopted at step 7. If you have never calibrated it reads zero, and the fix will carry the instrument’s error.

## 21   TABLE   Work the fix

Work the fix button.

The table reduces each sight against the DR, transports the lines to the common instant — that of the last sight — and crosses them all by least squares, in three passes.

## 22   TABLE   Read the result

The position, the shift from DR, the cocked hat, and the cut with its dilution.

The cocked hat proves nothing. It measures how well the lines agree, not how right the fix is. An error common to every sight — a badly calibrated index error — tightens it *while* moving the fix. One measured case: a 1.3-mile triangle around a fix six miles wrong.

## 23   TABLE   Read the wind, and adopt it

Under the fix: Computed wind 020° / 30 kt and, beside it, what you assumed. Adopt this wind button.

It is not given, it is found. The table compares your air position — where you would be with no wind at all, built from heading and airspeed alone — with your observed fix. The gap divided by the time elapsed is the wind you have been flying in. Adopting it pours it into the forecast wind, and the DR runs on with it.

Under five minutes since the last reset the table refuses and says so: over so short a run, one arcminute of error would invent several knots.

# THE PLOT — AND THE FIX ON THE CHART

## 24   TABLE   Open the plotting sheet

Plotting sheet button, next to “Work the fix”. If no fix has been worked yet, it works one before opening.

The sheet carries five things: your DR at the centre, the lines of position with their body’s name, the cocked hat, the fix, and the scale in miles. Nothing else.

It is not a chart: it is a position plotting sheet. Over the ocean at night a chart has nothing to show, and its scale is the wrong one — an intercept is a few miles, a chart covers a thousand. The square goes to what you assume, the circle to what you measured.

## 25   TABLE   Find the fix on the chart

It is already there: the table plotted it the moment you worked the fix. A green circle with its zulu time, and a line joining it to the DR it came from.

That line is what your sights corrected, so what the wind had made you drift. Working the fix again on the same book replaces the mark rather than stacking a second one on top.

## 26   TABLE   Reset the dead reckoning

Reset DR here button, in the fix band.

The DR jumps onto the fix and runs on from there; its previous track stays on the chart. Over a whole flight you read the story: the DR drifts, a fix pulls it back, it sets off again. The steady direction of those recoveries is the wind you had forecast wrong. The air plot is re-anchored too: the wind you measure at the next fix will be that of the leg to come.

## 27   TABLE   Open the debrief

Debrief button. It is the only door the truth comes out of, and it opens only once your work is handed in.

Truth: the fix was 1.5 NM off, the dead reckoning 39.9 — gain 38.4

# WHEN IT REFUSES — AND WHY

| WHAT YOU READ | WHAT TO DO |
| --- | --- |
| Visee de 18 s rejetee | Integrate at least thirty seconds. Below that the bubble’s mean is worthless. (Panel message, French.) |
| No dead reckoning | Connect the simulator — the DR sets itself on the ground — or work a fix and reset onto it. |
| One sight only | That makes a line of position, not a fix. Two are needed, three are better. |
| The aircraft is not on the ground | The airfield position is offered only on the ground. Airborne, typing a known position would be meaningless. |
| N more sights on slow bodies | Shoot closer to the meridian, under 3′/min. The table says how many are missing. |
| The run is too uniform | The knob-lag slope cannot be measured without a fast body. This is not a fault — it is the mark of a good run. |
| Wind not computed: less than five minutes | Wait. Wind is found over a leg, not over a turn. |
| TABLE HORS LIGNE | The bridge is not taken. Shoot anyway: the sights wait in the queue and leave at reconnection. The pending count shows beside it, in amber. |

# WHAT NOTHING WILL EVER TELL YOU

- Your true position. No aircraft marker, no latitude in the band, no chart following the aircraft. The middle of the screen would be the answer.

- Your ground speed and your track. They contain the drift, therefore the wind.

- The simulator’s real wind. Copying it into the forecast wind box would be enough for the DR to stop drifting, and there would be nothing left to find.

- Your instrument’s index error. It is drawn once and never shown: calibration is what reveals it, and your own collimation bias goes into it — the manual refuses to separate the two.

What stays visible is what can be read in a cockpit: altitude on the altimeter, heading on the compass, time on the watch. The rule fits in one sentence — what is read is allowed, what contains the wind is not.

Altitudes, azimuths and corrections come from the project’s own ephemeris core; the sextant reproduces the periscopic MIL-S-5807A and the MA-2, after AFM 51-40.